# Lab 7: Sequencing Control with the Handling Station

# Worksheet by Todd Johns, Dolan Stapleton, and Kyle Bennett

Defining the Logic

| - Step | - Condition | - Action | - Output |
| --- | --- | --- | --- |
| - 0 | - 1AI = 1 - 1S1 = 1 | - Home Position: Cylinder 2A retracted, Cylinder 3A Open, Cylinder 1A retracted, pushbutton not pressed | - 2AI = 0 2AO = 0 |
| - 1 | - 1S1 = 1 - 2S1 = 1 - Switch_0 flipped | - Cylinder 2A extends | - 2AI = 1 |
| - 2 | - 2AI = 1 - 2S2 = 1 | - Cylinder 3A closes | - 3A = 1 |
| - 3 | - 3A = 1 | - Cylinder 2A retracts | - 2AO = 1 |
| - 4 | - 2AO = 1 - 2S1 = 1 | - Cylinder 1A extends | - 1AO = 1 |
| - 5 | - 1AO = 1 - 1S2 = 1 | - Cylinder 2A extends | - 2AI = 1 |
| - 6 | - 2AI = 1 - 2S2 = 1 | - Cylinder 3A opens | - 3A = 0 |
| - 7 | - 3A = 0 | - Cylinder 2A retracts | - 2AO = 1 |
| - 8 | - 2AO = 1 - 2S1 = 1 | - Cylinder 1A retracts | - 1AI = 1 |

- NOTE: You may wish to also create a flowchart using Visio to help visualize the flow of the logic this program should perform.

Performing the Logic

Include a screenshot of your program below.